Bangabandhu Sheikh Mujibur Rahman Aviation and Aerospace University (BSMRAAU)

LAB REPORT

Experiment No: 01

Name of Experiment: MATLAB and Simulink Example

Problems and Solutions

Date of Experiment Submission: 19/09/2024

Student Name: Md. Mahdi Kamal Alif

Student ID: 22024019

Department: Department of Aerospace Engineering (Avionics)

Course Name: Modelling and Simulation Sessional
Course Code: AVE 4510

Submitted To: Asst. Prof. Md Samin Rahman

Asst. Prof. Dr. Md. Sakir Hossain

Remarks:

Modeling and Simulation Sessional

AVE: 4510

Experiment 1.1: Matrix Operations

Code:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 A = [3 4; 5 6]; B = [7 8; 9 10]; % Matrix Addition C = A + B; % Matrix Subtraction D = A - B; % Matrix Multiplication E = A * B; % Element-wise Multiplication F = A .* B; % Matrix Division G = A / B; % Display Results disp('Matrix Addition:'), disp(C) disp('Matrix Subtraction:'), disp(D) disp('Matrix Multiplication:'), disp(E) disp('Element-wise Multiplication:'), disp(F) disp('Matrix Division:'), disp(G)

Output:

Matrix Addition: 10 12 14 16 Matrix Subtraction: -4 -4 -4 -4 Matrix Multiplication: 57 64 89 100 Element-wise Multiplication: 21 32 45 60 Matrix Division: 3 -2 2 -1

Experiment 1.2: Solving Linear Equations:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 A = [1 2; 3 4]; b = [5; 6]; % Solve using inverse x1 = inv(A) * b; % Solve using backslash operator x2 = A \ b; % Display Results disp('Solution using inverse:'), disp(x1) disp('Solution using backslash operator:'), disp(x2)

Output:

Solution using inverse: -4.0000 4.5000 Solution using backslash operator: -4.0000 4.5000

Experiment 2.1: Plotting Data

%Author: Md. Mahdi Kamal Alif %ID: 22024019 x = 0:0.1:10; y = sin(x); figure; plot(x, y); title('Sine Wave'); xlabel('x'); ylabel('sin(x)'); grid on;

Output:

Experiment 2.2: Histogram

%Author: Md. Mahdi Kamal Alif %ID: 22024019 data = randn(1000, 1); figure; histogram(data); title('Histogram of Random Data'); xlabel('Data Values'); ylabel('Frequency'); grid on;

Output:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 Fs = 1000; % Sampling frequency T = 1/Fs; % Sampling period L = 1000; % Length of signal t = (0:L-1)*T; % Time vector % Create a signal S = 0.7*sin(2*pi*50*t) + sin(2*pi*120*t); % Compute the FFT Y = fft(S); % Compute the two-sided spectrum P2 and the single-sided spectrum P1 P2 = abs(Y/L); P1 = P2(1:L/2+1); P1(2:end-1) = 2*P1(2:end-1); % Define the frequency domain f f = Fs*(0:(L/2))/L; % Plot single-sided amplitude spectrum P1 figure; plot(f,P1) title('Single-Sided Amplitude Spectrum of S(t)') xlabel('f (Hz)') ylabel('¦P1(f)¦')

Experiment 3.1: FFT of a Signal

Output:

Experiment 4.1: Step Response of a Transfer Function

%Author: Md. Mahdi Kamal Alif %ID: 22024019 num = [1]; den = [1 3 2]; sys = tf(num, den); figure; step(sys); title('Step Response');

Output:

Experiment 5.1: Minimizing a Function

%Author: Md. Mahdi Kamal Alif %ID: 22024019 fun = @(x) (x-4)^2 + 6; x0 = 0; % Initial guess x = fminsearch(fun, x0); disp(‘The minimum value is at:’), disp(x)

Output:

The minimum value is at: 4.0000

Experiment 6.1: Solving ODEs

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Define the ODE ode = @(t, y) -2*y; % Initial condition y0 = 1; % Time span tspan = [0 5]; % Solve ODE [t, y] = ode45(ode, tspan, y0); % Plot results figure; plot(t, y); title('Solution of ODE'); xlabel('Time'); ylabel('y(t)'); grid on;

Output:

Experiment 7.1: Monte Carlo Simulation

%Author: Md. Mahdi Kamal Alif %ID: 22024019 %Number of simulations N = 10000; % Generate random samples samples = rand(N, 1); % Compute estimate of pi inside_circle = sum(samples.^2 + rand(N, 1).^2 <= 1); pi_estimate = 4 * inside_circle / N; disp('Estimated value of pi:'), disp(pi_estimate)

Output:

>> test1 Estimated value of pi: 3.1500 >> test2 Estimated value of pi: 3.1300 >> test3 Estimated value of pi: 3.1368

2D Plot:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 x=(0:pi/10:2*pi) y=sin(x) plot(x,y) title('2D Plot') xlabel('Time') ylabel('Amplitude')

Output:

3D plot:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 t=(-4:0.01:4) x=t.^2 y=4*t plot3(x,y,t) grid on xlabel('x-axis') ylabel('y-axis') zlabel('z-axis') title('3D Plot')

Output:

Experiment 10.1: Reading and Writing CSV Files

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Generate sample data data = rand(10, 3); % Write data to CSV csvwrite('data.csv', data); % Read data from CSV data_read = csvread('data.csv'); disp('Data read from CSV:'), disp(data_read)

Output:

Data read from CSV: 0.4596 0.7866 0.0222 0.9014 0.6951 0.5821 0.2594 0.4888 0.3880 0.9930 0.2136 0.9997 0.8682 0.8606 0.3072 0.6387 0.3510 0.0358 0.5015 0.8051 0.7321 0.2902 0.7039 0.1264 0.0098 0.0452 0.8136 0.9392 0.4641 0.6310 0.7792 0.3837 0.9319 0.6587 0.1746 0.6526 0.7498 0.5415 0.6724 0.2099

Impulse Function, Step Function and Ramp Function:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 t = (-2:0.01:10); impulse = t==0; unitstep = t>=0; ramp = t.*unitstep; % Plot impulse function figure; plot(t, impulse); title('Impulse Function'); xlabel('Time'); ylabel('Amplitude'); % Plot unit step function figure; plot(t, unitstep); title('Unit Step Function'); xlabel('Time'); ylabel('Amplitude'); % Plot ramp function figure; plot(t, ramp); title('Ramp Function'); xlabel('Time'); ylabel('Amplitude');

Output:

Linear Convolution:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 disp('enter the length of the first sequence m='); m=input(''); disp('enter the first sequence x[m]='); for i=1:m x(i)=input(''); end disp('enter the length of the second sequence n='); n=input(''); disp('enter the second sequence h[n]='); for j=1:n h(j)=input(''); end y=conv(x,h); figure; subplot(3,1,1); stem(x); ylabel ('amplitude---->'); xlabel('n--- >'); title('x(n) Vs n'); subplot(3,1,2); stem(h); ylabel('amplitude --->'); xlabel('n--- >'); title('h(n) Vs n'); subplot(3,1,3); stem(y); ylabel('amplitude --->'); xlabel('n--- >'); title('y(n) Vs n'); disp('linear convolution of x[m] and h[n] is y');

Output:

enter the length of the first sequence m= 4 enter the first sequence x[m]= 1 2 3 4 enter the length of the second sequence n= 5 enter the second sequence h[n]= 1 2 3 4 5 linear convolution of x[m] and h[n] is y >>

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Step 1: Define the resistance and current R = 10; % resistance in ohms I = 2; % current in amperes % Step 2: Calculate the voltage V = R * I; % Step 3: Display the result disp(['Voltage (V) = ', num2str(V), ' V']);

Ohm's Law Verification:

Output:

Voltage (V) = 20 V

2. Series RC Circuit Response

Output:

3. Parallel RLC Circuit

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Step 1: Define the values of R, L, and C R = 20; % resistance in ohms L = 2e-3; % inductance in henries C = 2e-6; % capacitance in farads % Step 2: Calculate the resonance frequency f_res = 1 / (2 * pi * sqrt(L * C)); disp(['Resonance Frequency (Hz) = ', num2str(f_res)]); % Step 3: Plot the impedance vs. frequency f = linspace(0, 2*f_res, 1000); Z = abs(R + 1./(1i*2*pi*f*C) + 1i*2*pi*f*L); plot(f, Z); xlabel('Frequency (Hz)'); ylabel('Impedance (Ohms)'); title('Impedance vs Frequency'); grid on;

Output:

Resonance Frequency (Hz) = 2516.4606

4. Diode Characteristics

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Step 1: Define the diode equation Is = 1e-12; % saturation current in amperes Vt = 0.025; % thermal voltage in volts % Step 2: Plot the current vs. voltage V = linspace(-0.7, 0.7, 1000); I = Is * (exp(V/Vt) - 1); plot(V, I); xlabel('Voltage (V)'); ylabel('Current (I)'); title('I-V Characteristics of a Diode'); grid on;

Output:

5. Transistor as a Switch

Output:

Full-wave Rectifier:

Output:

8. RC Low-Pass Filter

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Step 1: Define the values of R and C R = 2e3; % resistance in ohms C = 2e-6; % capacitance in farads % Step 2: Calculate the transfer function H = tf([1], [R*C 1]); % Step 3: Plot the frequency response bode(H); title('Frequency Response of RC Low-Pass Filter'); grid on;

Output:

9. RL High-Pass Filter

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Step 1: Define the values of R and L R = 2e3; % resistance in ohms L = 2e-3; % inductance in henries % Step 2: Calculate the transfer function H = tf([L 0], [L R]); % Step 3: Plot the frequency response bode(H); title('Frequency Response of RL High-Pass Filter'); grid on;

Output:

10. BJT Amplifier

Output:

4. Fourier Transform:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Fourier Transform of a sine wave Fs = 1000; % Sampling frequency t = 0:1/Fs:1-1/Fs; f = 5; % Frequency of sine wave x = sin(2*pi*f*t); X = fft(x); n = length(x); f = (0:n-1)*(Fs/n); % Frequency range magnitude = abs(X); plot(f,magnitude); axis([-500 1500 0 1000]); title('Fourier Transform'); xlabel('Frequency (Hz)'); ylabel('Magnitude');

Output:

6. AC Voltage Analysis:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % AC Voltage Analysis Fs = 1000; % Sampling frequency t = 0:1/Fs:1-1/Fs; f = 50; % Frequency of AC voltage V_peak = 20; % Peak voltage V_ac = V_peak * sin(2*pi*f*t); V_rms = rms(V_ac); disp(['RMS Voltage: ', num2str(V_rms), ' V']);

Output:

RMS Voltage: 14.1421 V

7. DC Motor Simulation:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % DC Motor Simulation J = 0.01; % Moment of inertia of the rotor b = 0.1; % Damping ratio of the mechanical system K = 0.01; % Electromotive force constant R = 1; % Electric resistance L = 0.5; % Electric inductance A = [0 1 0; 0 -b/J K/J; 0 -K/L -R/L]; B = [0; 0; 1/L]; C = [1 0 0]; D = 0; sys = ss(A,B,C,D); step(sys); title('DC Motor Step Response');

Output:

8. LED Characteristic Curve:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % LED Characteristic Curve V = 0:0.01:4; % Voltage range I = exp(V); % Simplified exponential I-V relationship plot(V, I); title('LED I-V Characteristic Curve'); xlabel('Voltage (V)'); ylabel('Current (A)');

Output:

9. BJT Common Emitter Amplifier:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % BJT Common Emitter Amplifier beta = 100; % Current gain R_C = 1e3; % Collector resistor in ohms V_CC = 15; % Supply voltage in volts I_B = 50e-6; % Base current in amperes I_C = beta * I_B; % Collector current in amperes V_CE = V_CC - I_C * R_C; % Collector-emitter voltage disp(['Collector-Emitter Voltage: ', num2str(V_CE), ' V']);

Output:

Collector-Emitter Voltage: 10 V

10. Diode Clipper Circuit:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Diode Clipper Circuit t = 0:0.001:1; % Time vector Vi = 5*sin(2*pi*20*t); % Input sine wave Vd = 0.7; % Diode forward voltage Vo = Vi; Vo(Vi > Vd) = Vd; Vo(Vi < -Vd) = -Vd; plot(t, Vi, t, Vo); title('Diode Clipper Circuit'); xlabel('Time (s)'); ylabel('Voltage (V)'); legend('Input Voltage', 'Output Voltage');

Output:

11. Transmission Line Impedance:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Transmission Line Impedance L = 1e-6; % Inductance per unit length (H/m) C = 2e-12; % Capacitance per unit length (F/m) Z0 = sqrt(L/C); % Characteristic impedance disp(['Characteristic Impedance: ', num2str(Z0), ' Ohms']);

Output:

Characteristic Impedance: 707.1068 Ohms

12. Phase Shift in RC Circuit:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Phase Shift in RC Circuit R = 1e3; % Resistance in ohms C = 1e-6; % Capacitance in farads f = 1000; % Frequency in Hz omega = 2*pi*f; % Angular frequency phi = atan(1/(omega*R*C)); % Phase shift disp(['Phase Shift: ', num2str(rad2deg(phi)), ' degrees']);

Output:

Phase Shift: 9.0431 degrees

13. Frequency Modulation (FM):

%Author: Md. Mahdi Kamal Alif %ID: 22024019 Fs = 10000; % Sampling frequency t = 0:1/Fs:1-1/Fs; Fc = 100; % Carrier frequency Fm = 5; % Modulating frequency Am = 1; % Modulating amplitude beta = 5; % Modulation index carrier = cos(2*pi*Fc*t); modulator = cos(2*pi*Fm*t); fm_signal = cos(2*pi*Fc*t + beta*sin(2*pi*Fm*t)); plot(t, fm_signal); title('Frequency Modulated Signal'); xlabel('Time (s)'); ylabel('Amplitude');

Output:

14. AM Modulation:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Amplitude Modulation Fs = 10000; % Sampling frequency t = 0:1/Fs:1-1/Fs; Fc = 100; % Carrier frequency Fm = 5; % Modulating frequency Am = 1; % Modulating amplitude carrier = cos(2*pi*Fc*t); modulator = cos(2*pi*Fm*t); am_signal = (1 + Am*modulator) .* carrier; plot(t, am_signal); title('Amplitude Modulated Signal'); xlabel('Time (s)'); ylabel('Amplitude');

Output:

15. Nyquist Plot:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Nyquist Plot sys = tf([1], [1 1 1]); nyquist(sys); title('Nyquist Plot');

Output:

17. Signal Sampling and Aliasing:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Signal Sampling and Aliasing Fs = 50; % Sampling frequency t = 0:1/Fs:1-1/Fs; f_signal = 20; % Signal frequency x = sin(2*pi*f_signal*t); stem(t, x); title('Signal Sampling and Aliasing'); xlabel('Time (s)'); ylabel('Amplitude');

Output:

18. Fast Fourier Transform (FFT):

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Fast Fourier Transform (FFT) Fs = 1000; % Sampling frequency t = 0:1/Fs:1-1/Fs; f = 5; % Frequency of sine wave x = sin(2*pi*f*t); X = fft(x); n = length(x); f = (0:n-1)*(Fs/n); % Frequency range magnitude = abs(X)/n; plot(f, magnitude); title('FFT of Sine Wave'); xlabel('Frequency (Hz)'); ylabel('Magnitude');

Output:

19. Bandpass Filter Design:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Bandpass Filter Design Fs = 1000; % Sampling frequency Fpass1 = 100; % First passband frequency Fpass2 = 200; % Second passband frequency N = 50; % Filter order bpFilt = designfilt('bandpassfir', 'FilterOrder', N, ... 'CutoffFrequency1', Fpass1, 'CutoffFrequency2', Fpass2, ... 'SampleRate', Fs); fvtool(bpFilt); title('Bandpass Filter Design');

Output:

20. Operational Amplifier (Op-Amp) Inverting Amplifier:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Op-Amp Inverting Amplifier Rin = 10e3; % Input resistor in ohms Rf = 100e3; % Feedback resistor in ohms Av = -Rf/Rin; % Voltage gain Vin = 1; % Input voltage in volts Vout = Av * Vin; % Output voltage disp(['Output Voltage: ', num2str(Vout), ' V']);

Output:

Output Voltage: -10 V

21. Butterworth Low Pass Filter Design:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Butterworth Low Pass Filter Design Fs = 1000; % Sampling frequency Fc = 200; % Cutoff frequency [b, a] = butter(4, Fc/(Fs/2)); % 4th order Butterworth filter freqz(b, a, [], Fs); title('Butterworth Low Pass Filter');

Output:

22. Transient Analysis of RC Circuit:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Transient Analysis of RC Circuit R = 1e3; % Resistance in ohms C = 1e-6; % Capacitance in farads sys = tf([1], [R*C 1]); impulse(sys); title('Transient Response of RC Circuit');

Output:

23. Voltage Divider Rule:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Voltage Divider Rule R1 = 10e3; % Resistor 1 in ohms R2 = 20e3; % Resistor 2 in ohms Vin = 10; % Input voltage in volts Vout = (R2 / (R1 + R2)) * Vin; % Output voltage disp(['Output Voltage: ', num2str(Vout), ' V']);

Output:

Output Voltage: 6.6667 V

24. Digital to Analog Conversion (DAC):

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Digital to Analog Conversion (DAC) digital_signal = [0 1 1 0 1 0 0 1]; analog_signal = filter(1, [1 -0.9], digital_signal); % Simple DAC model stem(analog_signal); title('Digital to Analog Conversion'); xlabel('Sample'); ylabel('Amplitude');

Output:

Output Voltage: 6.6667 V

25. Analog to Digital Conversion:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Analog to Digital Conversion (ADC) Fs = 1000; % Sampling frequency t = 0:1/Fs:1-1/Fs; analog_signal = sin(2*pi*50*t); % Analog signal digital_signal = round(analog_signal); % Simple ADC model stem(digital_signal); title('Analog to Digital Conversion'); xlabel('Sample'); ylabel('Amplitude');

Output:

26. Impedance of RLC Circuit:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Impedance of RLC Circuit R = 50; % Resistance in ohms L = 100e-3; % Inductance in henrys C = 10e-6; % Capacitance in farads f = 60; % Frequency in Hz omega = 2*pi*f; % Angular frequency Z = R + 1i*(omega*L - 1/(omega*C)); % Impedance disp(['Impedance: ', num2str(abs(Z)), ' Ohms']);

Output:

Impedance: 232.9875 Ohms

27. S-Parameters in RF Circuit:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % S-Parameters in RF Circuit s_params = sparameters('default.s2p'); % Load S-parameter file rfplot(s_params); title('S-Parameters');

Output:

28. Smith Chart:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Smith Chart z = linspace(0.1, 10, 100); % Normalized impedance values smithchart(z); title('Smith Chart');

Output:

29. Resonance in RLC Circuit:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Resonance in RLC Circuit L = 1e-3; % Inductance in henrys C = 100e-9; % Capacitance in farads f_resonance = 1/(2*pi*sqrt(L*C)); % Resonance frequency disp(['Resonance Frequency: ', num2str(f_resonance), ' Hz']);

Output:

Resonance Frequency: 15915.4943 Hz

30. Stability Analysis Using Routh-Hurwitz Criterion:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Stability Analysis Using Routh-Hurwitz Criterion coefficients = [1 3 3 1]; % Coefficients of the characteristic equation Routh_table = routh(coefficients); % Routh array disp('Routh Table:'); disp(Routh_table);

31. Power Spectral Density (PSD):

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Power Spectral Density (PSD) Fs = 1000; % Sampling frequency t = 0:1/Fs:1-1/Fs; x = cos(2*pi*100*t) + randn(size(t)); % Signal with noise [Pxx, f] = pwelch(x, [], [], [], Fs); plot(f, 10*log10(Pxx)); title('Power Spectral Density'); xlabel('Frequency (Hz)'); ylabel('Power/Frequency (dB/Hz)');

32. Step Response of a Transfer Function:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Step Response of a Transfer Function sys = tf([1], [1 2 1]); step(sys); title('Step Response');

Output:

33. Impulse Response of a Transfer Function:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Impulse Response of a Transfer Function sys = tf([1], [1 2 1]); impulse(sys); title('Impulse Response');

Output:

34. Phase Locked Loop (PLL) Simulation:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Phase Locked Loop (PLL) Simulation Fs = 1000; % Sampling frequency t = 0:1/Fs:1-1/Fs; F0 = 50; % Initial frequency input_signal = cos(2*pi*F0*t + pi/4); % Input signal [~, pll_output] = pll(input_signal, Fs); % Using a PLL function plot(t, input_signal, t, pll_output); title('Phase Locked Loop (PLL) Simulation'); xlabel('Time (s)'); ylabel('Amplitude'); legend('Input Signal', 'PLL Output');

35. Active Low Pass Filter:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Active Low Pass Filter R = 1e3; % Resistance in ohms C = 1e-6; % Capacitance in farads sys = tf([1], [R*C 1]); bode(sys); title('Active Low Pass Filter');

Output:

36. Digital Filter Design (FIR):

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Digital Filter Design (FIR) Fs = 1000; % Sampling frequency N = 50; % Filter order Fc = 200; % Cutoff frequency fir_coeff = fir1(N, Fc/(Fs/2)); freqz(fir_coeff, 1, [], Fs); title('FIR Digital Filter Design');

Output:

37. Digital Filter Design (IIR):

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Digital Filter Design (IIR) Fs = 1000; % Sampling frequency Fc = 200; % Cutoff frequency [b, a] = butter(4, Fc/(Fs/2)); freqz(b, a, [], Fs); title('IIR Digital Filter Design');

Output:

38. Root Locus Plot:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Root Locus Plot sys = tf([1], [1 3 3 1]); rlocus(sys); title('Root Locus Plot');

Output:

39. Signal Convolution:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Signal Convolution x = [1 2 3]; h = [1 1 1]; y = conv(x, h); stem(y); title('Signal Convolution'); xlabel('Sample'); ylabel('Amplitude');

Output:

40. Signal Correlation:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Signal Correlation x = [1 2 3 4]; y = [4 3 2 1]; correlation = xcorr(x, y); stem(correlation); title('Signal Correlation'); xlabel('Lag'); ylabel('Amplitude');

Output:

41. Power Factor Calculation:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Power Factor Calculation P = 100; % Real power in watts Q = 75; % Reactive power in VAR S = sqrt(P^2 + Q^2); % Apparent power in VA power_factor = P / S; disp(['Power Factor: ', num2str(power_factor)]);

Output:

Power Factor: 0.8

42. Step Response of a Control System:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Step Response of a Control System sys = tf([1], [1 3 3 1]); step(sys); title('Step Response of a Control System');

Output:

43. Nyquist Stability Criterion:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Nyquist Stability Criterion sys = tf([1], [1 3 3 1]); nyquist(sys); title('Nyquist Stability Criterion');

44. Fourier Series Approximation:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Fourier Series Approximation T = 2*pi; % Period t = linspace(0, T, 1000); % Time vector n_terms = 10; % Number of terms in the series square_wave = 0; for n = 1:2:n_terms square_wave = square_wave + (4/pi)*(sin(n*t)/n); end plot(t, square_wave); title('Fourier Series Approximation of Square Wave'); xlabel('Time'); ylabel('Amplitude');

Output:

45. Transmission Line Reflection Coefficient:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Transmission Line Reflection Coefficient Z0 = 50; % Characteristic impedance ZL = 100; % Load impedance reflection_coefficient = (ZL - Z0) / (ZL + Z0); disp(['Reflection Coefficient: ', num2str(reflection_coefficient)]);

Output:

Reflection Coefficient: 0.33333

46. State Space Representation:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % State Space Representation A = [0 1; -1 -1]; B = [0; 1]; C = [1 0]; D = 0; sys = ss(A, B, C, D); step(sys); title('State Space Representation');

47. Electrostatic Force Calculation:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Electrostatic Force Calculation q1 = 1e-6; % Charge 1 in coulombs q2 = 2e-6; % Charge 2 in coulombs r = 0.01; % Distance between charges in meters epsilon_0 = 8.854e-12; % Permittivity of free space F = (q1 * q2) / (4 * pi * epsilon_0 * r^2); % Force in newtons disp(['Electrostatic Force: ', num2str(F), ' N']);

Output:

Electrostatic Force: 179.7548 N

48. Electromagnetic Wave Propagation:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Electromagnetic Wave Propagation c = 3e8; % Speed of light in m/s f = 1e9; % Frequency in Hz lambda = c / f; % Wavelength in meters x = linspace(0, lambda, 1000); % Space vector E = sin(2 * pi * x / lambda); % Electric field plot(x, E); title('Electromagnetic Wave Propagation'); xlabel('Distance (m)'); ylabel('Electric Field (V/m)');

49. Frequency Response of a System:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Frequency Response of a System sys = tf([1], [1 3 3 1]); bode(sys); title('Frequency Response');

Output:

50. Pulse Width Modulation (PWM):

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Pulse Width Modulation (PWM) Fs = 1000; % Sampling frequency t = 0:1/Fs:1-1/Fs; duty_cycle = 0.5; % Duty cycle pwm_signal = square(2*pi*10*t, duty_cycle*100); % PWM signal plot(t, pwm_signal); title('Pulse Width Modulation (PWM)'); xlabel('Time (s)'); ylabel('Amplitude');

51. H-Bridge Circuit Simulation:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % H-Bridge Circuit Simulation V_dc = 12; % DC supply voltage in volts R_load = 10; % Load resistance in ohms t = 0:0.001:1; % Time vector V_out = V_dc * square(2*pi*10*t); % Output voltage with PWM control plot(t, V_out); title('H-Bridge Circuit Simulation'); xlabel('Time (s)'); ylabel('Voltage (V)');

52. PID Controller Design:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % PID Controller Design Kp = 1; % Proportional gain Ki = 1; % Integral gain Kd = 0.1; % Derivative gain sys = tf([1], [1 3 3 1]); % Plant transfer function pid_controller = pid(Kp, Ki, Kd); closed_loop_system = feedback(pid_controller * sys, 1); step(closed_loop_system); title('PID Controller Step Response');

53. Motor Speed Control Using PID:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Motor Speed Control Using PID J = 0.01; % Moment of inertia of the rotor b = 0.1; % Damping ratio K = 0.01; % Motor constant R = 1; % Electrical resistance L = 0.5; % Electrical inductance % Transfer function of the DC motor s = tf('s'); P_motor = K / (J*L*s^2 + (J*R + L*b)*s + (b*R + K^2)); % PID controller design Kp = 100; Ki = 200; Kd = 10; C = pid(Kp, Ki, Kd); % Closed-loop transfer function sys_cl = feedback(C*P_motor, 1); % Simulation t = 0:0.01:2; step(sys_cl, t); title('DC Motor Speed Control Using PID'); xlabel('Time (seconds)'); ylabel('Speed (rad/s)');

Output:

54. Phase Margin and Gain Margin:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Phase Margin and Gain Margin sys = tf([1], [1 3 3 1]); margin(sys); title('Phase Margin and Gain Margin');

Output:

55. State Observer Design:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % State Observer Design A = [0 1; -2 -3]; B = [0; 1]; C = [1 0]; D = 0; sys = ss(A, B, C, D); % Desired poles for the observer poles = [-5 -6]; L = place(A', C', poles)'; % Observer simulation t = 0:0.01:5; u = ones(size(t)); x0 = [0; 0]; [y, ~, x] = lsim(sys, u, t, x0); % State estimation x_hat = zeros(size(x)); for i = 2:length(t) x_hat(:, i) = x_hat(:, i-1) + 0.01 * (A*x_hat(:, i-1) + B*u(i) + L*(y(i-1) - C*x_hat(:, i-1))); end plot(t, x(1, :), 'b', t, x_hat(1, :), 'r--'); legend('True State', 'Estimated State'); title('State Observer Design'); xlabel('Time (s)'); ylabel('State');

56. Electromagnetic Interference (EMI) Simulation:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Electromagnetic Interference (EMI) Simulation Fs = 1000; % Sampling frequency t = 0:1/Fs:1-1/Fs; signal = sin(2*pi*50*t); % Original signal EMI = 0.5 * sin(2*pi*200*t); % Interference signal noisy_signal = signal + EMI; plot(t, signal, 'b', t, noisy_signal, 'r'); legend('Original Signal', 'Noisy Signal'); title('Electromagnetic Interference (EMI) Simulation'); xlabel('Time (s)'); ylabel('Amplitude');

Output:

57. Transmission Line Impedance Matching:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Transmission Line Impedance Matching Z0 = 50; % Characteristic impedance ZL = 100; % Load impedance matching_impedance = sqrt(Z0 * ZL); % Matching impedance disp(['Matching Impedance: ', num2str(matching_impedance), ' Ohms']);

Output:

Matching Impedance: 70.7107 Ohms

58. Switched-Mode Power Supply (SMPS) Simulation:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Switched-Mode Power Supply (SMPS) Simulation V_in = 12; % Input voltage in volts D = 0.5; % Duty cycle L = 1e-3; % Inductance in henries C = 1e-6; % Capacitance in farads R = 10; % Load resistance in ohms t = linspace(0, 0.01, 1000); % Time vector V_out = V_in * D * (1 - exp(-t / (L / R))); % Output voltage plot(t, V_out); title('Switched-Mode Power Supply (SMPS) Simulation'); xlabel('Time (s)'); ylabel('Voltage (V)');

61. Maximum Power Transfer Theorem:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Maximum Power Transfer Theorem R_source = 50; % Source resistance in ohms R_load = linspace(1, 100, 100); % Load resistance in ohms V_source = 10; % Source voltage in volts P_load = (V_source^2) * (R_load ./ (R_source + R_load).^2); % Power delivered to the load plot(R_load, P_load); title('Maximum Power Transfer Theorem'); xlabel('Load Resistance (Ohms)'); ylabel('Power Delivered (Watts)');

Output:

62. Mutual Inductance Calculation:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Mutual Inductance Calculation L1 = 10e-3; % Inductance of coil 1 in henrys L2 = 20e-3; % Inductance of coil 2 in henrys k = 0.5; % Coupling coefficient M = k * sqrt(L1 * L2); % Mutual inductance disp(['Mutual Inductance: ', num2str(M), ' H']);

Output:

Mutual Inductance: 0.0070711 H

63. Impedance Matching Using Smith Chart:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Impedance Matching Using Smith Chart z_load = 2 + 3j; % Normalized load impedance smithplot(z_load); title('Impedance Matching Using Smith Chart');

Output:

64. Sallen-Key Low Pass Filter:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Sallen-Key Low Pass Filter R = 1e3; % Resistance in ohms C = 1e-6; % Capacitance in farads sys = tf(1, [R^2*C^2 3*R*C 1]); bode(sys); title('Sallen-Key Low Pass Filter');

65. Thermistor Resistance Calculation:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Thermistor Resistance Calculation T = 25; % Temperature in Celsius T0 = 25; % Reference temperature in Celsius R0 = 10e3; % Resistance at T0 in ohms B = 3950; % Beta coefficient R = R0 * exp(B * ((1 / (T + 273.15)) - (1 / (T0 + 273.15)))); disp(['Thermistor Resistance: ', num2str(R), ' Ohms']);

Output:

Thermistor Resistance: 10000 Ohms

66. Phase Shifter Circuit:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Phase Shifter Circuit f = 1000; % Frequency in Hz R = 1e3; % Resistance in ohms C = 1e-6; % Capacitance in farads omega = 2 * pi * f; phi = atan(1 / (omega * R * C)); % Phase shift in radians disp(['Phase Shift: ', num2str(phi * (180/pi)), ' degrees']);

Output:

Phase Shift: 9.0431 degrees

67. Schmitt Trigger Circuit:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Schmitt Trigger Circuit Vcc = 5; % Supply voltage in volts R1 = 1e3; % Resistance R1 in ohms R2 = 10e3; % Resistance R2 in ohms Vin = linspace(-Vcc, Vcc, 1000); % Input voltage sweep Vout = zeros(size(Vin)); Vh = Vcc * (R2 / (R1 + R2)); % High threshold voltage Vl = -Vcc * (R2 / (R1 + R2)); % Low threshold voltage for i = 2:length(Vin) if Vin(i) > Vh Vout(i) = Vcc; elseif Vin(i) < Vl Vout(i) = -Vcc; else Vout(i) = Vout(i-1); end end plot(Vin, Vout); title('Schmitt Trigger Circuit'); xlabel('Input Voltage (V)'); ylabel('Output Voltage (V)');

Output:

68. Band Pass Filter Design:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Band Pass Filter Design R = 1e3; % Resistance in ohms L = 1e-3; % Inductance in henrys C = 1e-6; % Capacitance in farads sys = tf([R*C 0], [L*C R*C 1]); bode(sys); title('Band Pass Filter Design');

Output:

69. Transient Response Analysis:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Transient Response Analysis wn = 5; % Natural frequency zeta = 0.5; % Damping ratio sys = tf([wn^2], [1 2*zeta*wn wn^2]); step(sys); title('Transient Response Analysis');

Output:

70. Delta-Wye Transformation:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Delta-Wye Transformation Zab = 50; % Impedance between points A and B Zbc = 50; % Impedance between points B and C Zca = 50; % Impedance between points C and A Z1 = Zab * Zbc / (Zab + Zbc + Zca); Z2 = Zbc * Zca / (Zab + Zbc + Zca); Z3 = Zca * Zab / (Zab + Zbc + Zca); disp(['Z1: ', num2str(Z1), ' Ohms']); disp(['Z2: ', num2str(Z2), ' Ohms']); disp(['Z3: ', num2str(Z3), ' Ohms']);

Output:

Z1: 16.6667 Ohms Z2: 16.6667 Ohms Z3: 16.6667 Ohms

71. RC Integrator Circuit

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % RC Integrator Circuit R = 1e3; % Resistance in ohms C = 1e-6; % Capacitance in farads sys = tf([1], [R*C 1]); bode(sys); title('RC Integrator Circuit');

Output:

72. Buck Converter Simulation:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Buck Converter Simulation Vin = 12; % Input voltage in volts Vout = 5; % Desired output voltage in volts D = Vout / Vin; % Duty cycle t = 0:0.0001:0.01; % Time vector V = Vin * D * square(2*pi*1e3*t); % Output voltage waveform plot(t, V); title('Buck Converter Simulation'); xlabel('Time (s)'); ylabel('Voltage (V)');

Output:

73. Bode Plot Analysis

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Bode Plot Analysis sys = tf([1], [1 3 3 1]); bode(sys); title('Bode Plot Analysis');

Output:

74. Root Mean Square (RMS) Calculation:

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Root Mean Square (RMS) Calculation t = linspace(0, 2*pi, 1000); x = sin(t); RMS = sqrt(mean(x.^2)); disp(['RMS Value: ', num2str(RMS)]);

Output:

RMS Value: 0.70675

75. AC Voltage Regulation

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % AC Voltage Regulation V_no_load = 230; % No-load voltage in volts V_full_load = 220; % Full-load voltage in volts regulation = ((V_no_load - V_full_load) / V_full_load) * 100; disp(['Voltage Regulation: ', num2str(regulation), '%']);

Output:

Voltage Regulation: 4.5455%

76. Spectral Analysis Using FFT

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Spectral Analysis Using FFT Fs = 1000; % Sampling frequency t = 0:1/Fs:1-1/Fs; x = cos(2*pi*100*t) + 0.5*cos(2*pi*200*t); % Signal with two frequencies X = fft(x); f = (0:length(X)-1)*Fs/length(X); % Frequency vector plot(f, abs(X)); title('Spectral Analysis Using FFT'); xlabel('Frequency (Hz)'); ylabel('Magnitude');

Output:

77. Resonance in RLC Circuit

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Resonance in RLC Circuit R = 10; % Resistance in ohms L = 1e-3; % Inductance in henrys C = 1e-6; % Capacitance in farads f_resonance = 1 / (2*pi*sqrt(L*C)); % Resonant frequency disp(['Resonant Frequency: ', num2str(f_resonance), ' Hz']); f = linspace(0, 2*f_resonance, 1000); Z = sqrt(R^2 + (2*pi*f*L - 1./(2*pi*f*C)).^2); % Impedance plot(f, Z); title('Resonance in RLC Circuit'); xlabel('Frequency (Hz)'); ylabel('Impedance (Ohms)');

Output:

78. Synchronous Motor Simulation

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Synchronous Motor Simulation V = 230; % Supply voltage in volts f = 50; % Supply frequency in Hz P = 4; % Number of poles N_sync = 120*f/P; % Synchronous speed in RPM t = linspace(0, 1, 1000); theta = 2*pi*N_sync/60 * t; % Rotor angle in radians I = V * sin(theta); % Induced current plot(t, I); title('Synchronous Motor Simulation'); xlabel('Time (s)'); ylabel('Current (A)');

Output:

79. Power System Load Flow Analysis

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Power System Load Flow Analysis using Newton-Raphson Method % Define system parameters Ybus = [ 10-20j -5+10j -5+10j; -5+10j 8-16j -3+6j; -5+10j -3+6j 8-16j ]; % Y-bus matrix P = [-1.2; 1.0; 0.5]; % Active power demand (per unit) Q = [-0.5; 0.3; 0.2]; % Reactive power demand (per unit) V = [1; 1; 1]; % Initial guess for voltage magnitudes theta = [0; 0; 0]; % Initial guess for voltage angles % Newton-Raphson iteration max_iter = 10; tol = 1e-6; for iter = 1:max_iter % Calculate power mismatches P_calc = real(V .* (Ybus * (V .* exp(1j*theta)))); Q_calc = imag(V .* (Ybus * (V .* exp(1j*theta)))); dP = P - P_calc; dQ = Q - Q_calc; mismatch = [dP; dQ]; % Check for convergence if norm(mismatch) < tol break; end % Jacobian matrix calculation J11 = diag(V) * real(Ybus * diag(V .* exp(1j*theta))); J12 = diag(V) * imag(Ybus * diag(V .* exp(1j*theta))); J21 = diag(V) * imag(Ybus * diag(V .* exp(1j*theta))); J22 = -diag(V) * real(Ybus * diag(V .* exp(1j*theta))); J = [J11 J12; J21 J22]; % Update voltage magnitudes and angles correction = inv(J) * mismatch; dtheta = correction(1:length(P)); dV = correction(length(P)+1:end); theta = theta + dtheta; V = V + dV; end disp('Voltage magnitudes (pu):'); disp(V); disp('Voltage angles (degrees):'); disp(rad2deg(theta));

Output:

Voltage magnitudes (pu): 1.0e+14 * -1.3511 -1.3511 -1.3511 Voltage angles (degrees): 1.0e+15 * -3.8706 -3.8706 -3.8706

80. Diode Clipper Circuit

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Diode Clipper Circuit V_in = linspace(-10, 10, 1000); % Input voltage range V_clip = 3; % Clipping voltage V_out = zeros(size(V_in)); for i = 1:length(V_in) if V_in(i) > V_clip V_out(i) = V_clip; elseif V_in(i) < -V_clip V_out(i) = -V_clip; else V_out(i) = V_in(i); end end plot(V_in, V_out); title('Diode Clipper Circuit'); xlabel('Input Voltage (V)'); ylabel('Output Voltage (V)');

Output:

81. Voltage Doubler Circuit

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Voltage Doubler Circuit V_in = 5; % Input AC voltage peak f = 60; % Frequency in Hz t = linspace(0, 1/f, 1000); % Time vector V_out = 2 * V_in * abs(sin(2*pi*f*t)); % Output voltage waveform plot(t, V_out); title('Voltage Doubler Circuit'); xlabel('Time (s)'); ylabel('Voltage (V)');

Output:

82. Three-Phase Power Calculation

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Three-Phase Power Calculation V_phase = 230; % Phase voltage in volts I_phase = 10; % Phase current in amperes pf = 0.8; % Power factor P = 3 * V_phase * I_phase * pf; % Total real power Q = 3 * V_phase * I_phase * sqrt(1 - pf^2); % Total reactive power disp(['Total Real Power: ', num2str(P), ' W']); disp(['Total Reactive Power: ', num2str(Q), ' VAR']);

Output:

Total Real Power: 5520 W Total Reactive Power: 4140 VAR

83. MOSFET Characteristics Plot

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % MOSFET Characteristics Plot Vds = linspace(0, 10, 100); % Drain-source voltage range Vgs = [2, 4, 6]; % Gate-source voltages for Vg = Vgs Id = (Vg > 1) .* ((Vg - 1) .* (Vds < Vg - 1) + (Vg - 1)^2/2 .* (Vds >= Vg - 1)); plot(Vds, Id); hold on; end hold off; title('MOSFET I-V Characteristics'); xlabel('V_{DS} (V)'); ylabel('I_{D} (A)'); legend('V_{GS} = 2V', 'V_{GS} = 4V', 'V_{GS} = 6V');

Output:

84. Stepper Motor Control

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Stepper Motor Control steps = 100; % Number of steps step_angle = 1.8; % Step angle in degrees angle = 0; % Initial angle angles = zeros(1, steps); for i = 1:steps angle = angle + step_angle; angles(i) = angle; end plot(1:steps, angles); title('Stepper Motor Control'); xlabel('Step'); ylabel('Angle (degrees)');

Output:

85. Photodiode IV Characteristics

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Photodiode IV Characteristics V = linspace(-2, 2, 100); % Voltage range I_dark = 1e-9; % Dark current in amperes n = 1; % Ideality factor Is = 1e-12; % Saturation current in amperes k = 1.38e-23; % Boltzmann constant T = 300; % Temperature in Kelvin q = 1.6e-19; % Electron charge I = Is * (exp((q*V)/(n*k*T)) - 1) + I_dark; % Current plot(V, I); title('Photodiode IV Characteristics'); xlabel('Voltage (V)'); ylabel('Current (A)');

Output:

86. Battery Charging Simulation

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Battery Charging Simulation V_batt = 12; % Battery voltage in volts I_charge = 2; % Charging current in amperes C = 60; % Battery capacity in Ah t = linspace(0, C/I_charge, 1000); % Time vector V = V_batt * (1 - exp(-t / (C/I_charge))); % Voltage during charging plot(t, V); title('Battery Charging Simulation'); xlabel('Time (hours)'); ylabel('Voltage (V)');

Output:

88. Series RLC Circuit Analysis

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Series RLC Circuit Analysis R = 10; % Resistance in ohms L = 1e-3; % Inductance in henrys C = 1e-6; % Capacitance in farads V = 1; % Voltage step input in volts sys = tf([1], [L*C R*C 1]); t = linspace(0, 0.01, 1000); step_response = step(V*sys, t); plot(t, step_response); title('Series RLC Circuit Analysis'); xlabel('Time (s)'); ylabel('Voltage (V)');

Output:

89. Light Emitting Diode (LED) Characteristics

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % LED Characteristics V = linspace(0, 3, 100); % Voltage range I = 1e-9 * (exp(V / 0.026) - 1); % Current plot(V, I); title('LED I-V Characteristics'); xlabel('Voltage (V)'); ylabel('Current (A)');

Output:

90. Full-Wave Rectifier Simulation

%Author: Md. Mahdi Kamal Alif %ID: 22024019 % Full-Wave Rectifier Simulation t = linspace(0, 0.1, 1000); % Time vector V_in = sin(2 * pi * 50 * t); % Input AC voltage V_out = abs(V_in); % Full-wave rectified voltage plot(t, V_in, t, V_out); title('Full-Wave Rectifier Simulation'); xlabel('Time (s)'); ylabel('Voltage (V)'); legend('Input Voltage', 'Output Voltage');

Output: